MMT Studio 5G

User & Reference Manual

5G SA Core  +  Tester  +  Orchestration

A complete guide — what each component is, why it exists, and what you can do with it

MakeMyTechnology

Release v2026.06.2

Licensed under AGPL-3.0-or-later  ·  info@makemytechnology.com

Table of Contents

1. Introduction	4

1.1 The three components	4

1.2 Who this manual is for	4

1.3 What makes it different	4

2. The Core — the 5G network	5

2.1 What is the Core?	5

2.2 Why we use it	5

2.3 Network functions it provides	5

Control plane	5

User plane	6

Interworking & services	6

2.4 How the Core is built (design patterns)	6

2.5 What you can do with the Core	6

2.6 The end-to-end flow it runs	7

3. The Tester — the validation system	8

3.1 What is the Tester?	8

3.2 Why we use it	8

3.3 What it can emulate and test	8

3.4 The test suites	8

3.5 What you can do with the Tester	9

3.6 Example commands	9

4. How the Core and Tester work together	10

4.1 The role of Orchestrate	10

4.2 The data path	10

4.3 Bringing the studio up	10

Linux	10

Windows 11 (WSL2)	10

4.4 Choosing a role	10

5. Installation & first run	11

5.1 Prerequisites	11

5.2 Install on Linux	11

5.3 Install on Windows	11

5.4 After bring-up	11

5.5 Building each component separately	11

6. Standards & spec compliance	12

7. Status & licensing	12

8. Glossary	12

# 1. Introduction

MMT Studio 5G is an open-source, 3GPP-aligned 5G System Architecture (SA) reference distribution from MakeMyTechnology. It lets you stand up a complete, standards-grounded 5G mobile core network on a single machine and then exercise it with a built-in tester that emulates real radios and phones — all without any commercial equipment, SIM cards, or radio hardware.

In a real mobile network there are two halves: the network (the operator’s core that authenticates subscribers, sets up data sessions, and routes traffic) and the devices that connect to it (base stations and handsets). MMT Studio gives you both halves in software so you can learn, develop, and validate 5G features end-to-end on a laptop or server.

## 1.1 The three components

The distribution ships three sibling components laid out under the repository root:

| Component | Directory | What it is |
| --- | --- | --- |
| Core | core/ | The 5G Core network itself, written in Go + C/DPDK. This is the “network” side — AMF, SMF, UPF, AUSF, UDM, PCF, NRF, IMS and more. |
| Tester | tester/ | A Python + Robot Framework test system that emulates gNBs (base stations) and UEs (phones) to drive and validate the Core. |
| Orchestrate | orchestrate/ | Docker-Compose “glue” that brings the Core and Tester up together on one host with a single command. |

In short: the Core is the thing under test, the Tester is the thing that tests it, and Orchestrate wires them together so they can talk.

## 1.2 Who this manual is for

- Engineers and students learning how a 5G SA network works, procedure by procedure.

- Developers building or modifying network functions and needing a reference core to run against.

- Test and validation engineers who want a ready-made, spec-citing conformance harness.

- Private-network operators evaluating 5G features before committing to vendor equipment.

## 1.3 What makes it different

- Spec-grounded. Every meaningful procedure carries a §-citation to a 3GPP Technical Specification or IETF RFC, and a built-in verifier (speccheck) confirms every cited clause actually exists in the local spec PDFs.

- Real protocols, real crypto. Kernel SCTP for NGAP signalling, a DPDK-accelerated user plane, and byte-exact Milenage / 5G key-derivation cryptography — not mocked stubs.

- Self-contained. DPDK source, Python runtime, codecs and crypto libraries all ship in-repo. There are effectively zero external dependencies to chase.

- Reference / research grade. Suitable for lab interop, conformance trials, R&D, and university coursework — not (yet) certified for commercial production.

# 2. The Core — the 5G network

## 2.1 What is the Core?

The Core (directory core/, runtime name sacore) is a complete 5G Standalone core network implemented as a single Go binary with a C/DPDK fast path for user traffic. When people say “the 5G network” from an operator’s point of view, they mean the core: the collection of network functions that authenticate a subscriber, decide what data services they may use, set up the data tunnels, enforce policy and charging, and carry the actual user traffic.

It exposes a web GUI on port 5000, speaks real NGAP / S1AP / NAS signalling over real kernel SCTP, and runs a DPDK-accelerated UPF data plane. It ships with over 1,245 verified specification citations grounded in local 3GPP / IETF documents.

## 2.2 Why we use it

- To have a working, inspectable 5G core you fully control — every byte on the wire and every state transition is visible and spec-cited.

- To develop and debug network functions without needing a commercial core or radio gear.

- To study 3GPP procedures against a real implementation that refuses to take non-compliant shortcuts.

- To run private 5G experiments and feature trials before any vendor adoption.

## 2.3 Network functions it provides

The Core implements the standard 5G control-plane and user-plane functions, plus interworking and a broad set of services. Each is a real network function (NF) with its own finite state machine where the lifecycle is non-trivial.

### Control plane

| NF | Full name | What it does |
| --- | --- | --- |
| AMF | Access & Mobility Mgmt Function | Anchors signalling from the radio; handles registration, authentication orchestration, mobility, and NAS security. |
| SMF | Session Management Function | Sets up, modifies and tears down PDU (data) sessions; controls the UPF over PFCP. |
| AUSF | Authentication Server Function | Runs 5G-AKA authentication using real Milenage vectors. |
| UDM / UDR | Unified Data Mgmt / Repository | Holds subscriber data (keys, profiles) and serves it to other NFs. |
| PCF | Policy Control Function | Decides QoS and policy rules per session (SM Policy). |
| NSSF | Network Slice Selection Function | Selects the network slice for a subscriber. |
| CHF | Charging Function | Online/offline charging (TS 32.290/291). |
| NRF | Network Repository Function | Service registration and discovery between NFs. |
| NWDAF | Network Data Analytics Function | Collects data and exposes analytics. |
| SMSF / LMF / NSACF / GMLC / AF | Supporting functions | SMS, location, slice admission control, and application exposure. |

### User plane

The UPF (User Plane Function) is where actual user data flows. It is a Go control plane bridged via cgo to a C/DPDK fast path (libupf_dp.so), implementing the full TS 29.244 PFCP surface (session Establish / Modify / Delete / Query) and GTP-U encapsulation. This is what carries the megabits per second once a phone’s data session is up.

### Interworking & services

- EPC interworking — a 4G MME with S1AP + EMM + ESM and the N26 interface to the AMF, for 4G↔5G mobility.

- IMS / voice — SIP / CSCF / MRFP for VoNR and ViNR (voice and video over 5G).

- Services — MCX (mission-critical push-to-talk / data / video), eSIM (SM-DP+), V2X, PROSE, SEAL, UAS, USSD, FrMCS.

- Edge, safety, security, IoT — MEC, emergency / IOPS / PWS, lawful intercept and DPI, NB-IoT / RedCap / Ambient IoT.

## 2.4 How the Core is built (design patterns)

A handful of enforced patterns keep the implementation auditable and standards-true:

- Per-NF finite state machines. Each NF with a real lifecycle has its own state/event/fsm triplet, so procedure collisions are caught at the transition table rather than with ad-hoc branching.

- Single-owner NAS security. All integrity, ciphering, NAS COUNT and key derivation live in one package (nf/amf/security/); handlers never touch keys directly.

- Spec citations in code. Every non-trivial handler quotes the exact 3GPP clause it implements; unimplemented requirements ship as TS-numbered TODO blocks at the call site.

- Runtime citation verifier (speccheck). Walks every “TS NN.NNN §X.Y” string in the code, resolves it against a local PDF, and fails the build on a dangling cite.

## 2.5 What you can do with the Core

- Run a full registration + authentication + PDU-session + data-flow + release cycle end-to-end with real codecs and crypto.

- Inspect 69 HTML panels and call 549+ REST endpoints from the web GUI on port 5000.

- Watch live NGAP on :38412 (SCTP) and PFCP on :8805 (UDP).

- Switch the database between SQLite (default) and PostgreSQL.

- Benchmark the control plane (16/32/64/128-UE registration) and the data plane (per-UE line-rate throughput).

- Build a Debian package or Docker image, or run as a systemd service.

## 2.6 The end-to-end flow it runs

A complete attach exercises the per-UE GMM, per-UE NGAP, and per-session 5GSM state machines together:

gNB → NG Setup Request            (NGAP FSM: IDLE → ACTIVE)

AMF → AUSF → UDM → UDR          (subscriber K/OP lookup)

AUSF → Milenage f1/f2345          (real AES-based auth vector)

AMF → Authentication Request      (RAND + AUTN)

AMF → Security Mode Command       (NAS ciphering + integrity on)

AMF → Registration Accept

UE  → PDU Session Establishment   (via UL NAS Transport with PTI)

SMF → PCF SMPolicy                (policy decision)

SMF → UPF (PFCP N4)               (PDR/FAR/QER/URR installed)

UPF → GTP-U tunnel up             (user-plane data flows)

# 3. The Tester — the validation system

## 3.1 What is the Tester?

The Tester (directory tester/, runtime name satester) is a Python + Robot Framework system that emulates the radio side of the network: it pretends to be one or more gNBs (5G base stations) and a pool of UEs (phones / devices). It connects to the Core exactly as real equipment would — over SCTP/NGAP for signalling and GTP-U/UDP for data — and drives it through real procedures, then checks that the Core responds in a standards-compliant way.

It has its own web UI (in the studio it runs on host port 5001) and a CLI, and bundles its own Python runtime, iperf3, libsctp and every package, so it has zero external dependencies.

## 3.2 Why we use it

- To prove the Core actually works — not just that it starts, but that each 3GPP procedure completes correctly.

- To catch regressions: tests are written to FAIL on any non-compliant response, with no fallbacks or workarounds.

- To exercise the Core at scale (many UEs, rapid attach/detach, concurrent traffic) without buying handsets or radios.

- To generate real user-plane traffic (iperf3, RTP voice/video) and measure throughput, QoS and latency.

## 3.3 What it can emulate and test

| Capability | What it covers |
| --- | --- |
| 5G NAS / NGAP | Registration, Authentication (5G-AKA), Security Mode, PDU Session setup. |
| GTP-U data plane | TUN interfaces, QoS classification, per-UE policy routing. |
| IMS / VoNR / ViNR | SIP REGISTER, INVITE, re-INVITE, REFER, BYE with IMS-AKA. |
| Conference calls | 3-way to 6-way audio/video via MRFP (TS 24.147). |
| Traffic engine | iperf3 UL/DL, RTP audio (AMR-WB) and video (H.264). |
| Handover | N2 handover with GTP-U tunnel switching. |
| Network slicing | eMBB / URLLC / MIoT slice testing. |
| Positioning | E-CID, Multi-RTT, DL-TDOA, A-GNSS, geofencing (TS 23.273). |
| IoT | NB-IoT PSM/eDRX, CP CIoT, NIDD/SCEF, Ambient IoT tags. |
| NTN | Satellite constellation, coverage, timing advance, feeder links (TS 38.821). |

## 3.4 The test suites

The Tester ships with roughly 545 test cases (313 Robot + 232 Python OAM) organised into Robot Framework suites grouped by domain. A representative set:

| # | Suite | Tests | Coverage (3GPP) |
| --- | --- | --- | --- |
| 01 | Registration | 6 | TS 24.501 §5.5.1, TS 33.501 §6.1.3 |
| 02 | PDU Session | 4 | TS 24.501 §6.4, TS 29.244 |
| 04 | Stress | 16 | Multi-UE registration, rapid attach/detach |
| 05 | NG Setup | 16 | TS 38.413 §8.7 |
| 06 | Authentication | 12 | 5G-AKA, SUCI, ECIES |
| 07 | Traffic | 13 | UDP/TCP UL/DL, AMBR, MBR, GBR |
| 08 | IMS | 17 | VoNR, ViNR, conference, mid-call upgrade |
| 10 | IMS Scale | 16 | Multi-UE IMS calls |
| 12 | Handover | 6 | N2 handover, Xn |
| 16 | Slicing | 10 | eMBB / URLLC / MIoT slices |
| 17 | Positioning | 10 | E-CID, RTT, TDOA, GNSS, geofence |
| 18 | IoT | 15 | NB-IoT, NIDD/SCEF, Ambient IoT |
| 19 | NTN | 12 | Satellite, coverage, timing, TAI |

## 3.5 What you can do with the Tester

- Run a single suite or a full regression from the CLI or web UI, and produce HTML / JUnit reports.

- Configure gNB and UE profiles entirely from the web GUI — PLMN, TAC, gNB ID, slices, IMSI, MSISDN, K/OPc, SUPI vs SUCI.

- Point the tester at a local Core or a remote Core by IP, for split-host testing.

- Drive real voice/video calls and conferences, then measure media quality.

- Generate UL/DL/bidirectional traffic at target rates and confirm QoS enforcement.

- Analyse pass-rate trends over time and use the optional AI engine for analysis.

## 3.6 Example commands

# Run the IMS suite and produce an HTML report

python3 -m src.cli run --suite 08_ims --report html

# Full regression with JUnit output and a CI exit code

python3 -m src.cli run --report html junit --exit-code

# Look at pass-rate trends

python3 -m src.cli analysis pass-rate

# 4. How the Core and Tester work together

## 4.1 The role of Orchestrate

Orchestrate (directory orchestrate/) is the Docker-Compose glue that brings the whole studio up on one host with a single command. It runs three containers on one custom bridge network (mmtnet, 172.30.0.0/24):

| Service | Role | Bridge IP / UI |
| --- | --- | --- |
| sacore | The Go Core (AMF/SMF/UPF); owns the upfgtp TUN. | 172.30.0.10 · http://localhost:5000 |
| satester | The Python Tester (master) + gNB simulator. | 172.30.0.20 · http://localhost:5001 |
| satraffic | Traffic-agent slave sharing sacore’s netns, so DL traffic traverses the UPF. | shares 172.30.0.10:9100 |

A custom bridge with static IPs is used (rather than host networking) so the Core’s UPF and the Tester’s gNB simulator can both bind 0.0.0.0:2152 (GTP-U) without colliding, while container-to-container SCTP still works without any NAT in the way.

## 4.2 The data path

The two halves connect over the same interfaces real equipment uses:

- Signalling: Tester gNB-sim ↔ Core AMF over SCTP/NGAP on port 38412.

- User data: Tester ↔ Core UPF over GTP-U/UDP on port 2152, with iperf3 / RTP riding inside the tunnels.

## 4.3 Bringing the studio up

### Linux

cd mmt-studio-5g6g/orchestrate

./run_studio.sh            # bring up the full stack (core + tester)

./run_studio.sh status     # ps + watcher status

./run_studio.sh logs       # follow logs

./run_studio.sh down       # stop + remove containers + bridge

### Windows 11 (WSL2)

cd mmt-studio-5g6g\orchestrate

run_studio.bat             # bring up + per-test Wireshark watcher

run_studio.bat status      # compose ps + watcher status

run_studio.bat logs        # follow logs

run_studio.bat down        # stop stack + Wireshark watcher

By default both platforms open Wireshark live on every test run so you can watch the packets; pass --no-wireshark (Linux) or -NoWireshark (Windows) to opt out.

## 4.4 Choosing a role

The installer and run script accept --role={both|core|tester}. Use core to run only the Core side, tester to run only the Tester (add --core-host=<IP> to point at a remote Core), or both for the full studio.

# 5. Installation & first run

## 5.1 Prerequisites

- Linux: Ubuntu 22.04+ / Debian 12+ with Docker Engine 24+ (bare metal, VirtualBox guest, or WSL2). VirtualBox guests need ≥ 4 GiB RAM and 4 vCPUs.

- Windows: Windows 11 (or Windows 10 build 19041+) with WSL2.

- Hugepages: the UPF data plane uses DPDK and needs at least 512 × 2 MiB hugepages. Both installers allocate and persist these automatically.

## 5.2 Install on Linux

git clone https://github.com/Makemytechnology/mmt-studio-5g6g.git

mmt-studio-5g6g/orchestrate/install.sh --role=both

The installer self-elevates and asks for your sudo password when it needs root.

## 5.3 Install on Windows

git clone https://github.com/Makemytechnology/mmt-studio-5g6g.git

mmt-studio-5g6g\orchestrate\install_on_windows.bat

No need to “Run as Administrator” — the .bat self-elevates with a UAC prompt. If Windows asks for a reboot after the WSL2 step, reboot and re-run.

## 5.4 After bring-up

Once the stack is up, the studio is reachable on the host at:

| Service | Web UI | Bridge IP |
| --- | --- | --- |
| sacore (Core) | http://localhost:5000 | 172.30.0.10 |
| satester (Tester) | http://localhost:5001 | 172.30.0.20 |

In the Tester web UI, set the gNB profile to AMF IP 172.30.0.10, AMF SCTP port 38412, UPF IP 172.30.0.10, GTP-U port 2152 — then run a suite and watch the Core respond.

## 5.5 Building each component separately

| Component | Build command |
| --- | --- |
| core/ | cd core && go build ./... (Go 1.22+); DPDK under libs/dpdk-25.11/ uses meson/ninja. |
| tester/ | cd tester && python3 -m venv .venv && .venv/bin/pip install -r build/requirements.txt |
| orchestrate/ | cd orchestrate && ./run_studio.sh up (builds container images on first run) |

# 6. Standards & spec compliance

Every meaningful procedure in both the Core and the Tester carries a §-citation to a 3GPP TS or IETF RFC, anchored to local PDF/RFC files. A speccheck tool verifies that every cited section actually exists in the referenced document, and fails the build if a citation is dangling or mis-numbered. The Core grounds 1,245 / 1,245 citations.

Selected standards covered:

- N1/N2/N3/N4 — TS 23.501, TS 23.502, TS 24.501, TS 29.281, TS 29.244

- NGAP — TS 38.413;  S1AP — TS 36.413

- Policy / Charging — TS 23.503, TS 29.512, TS 32.290, TS 32.291

- IMS / SIP — TS 23.228, TS 24.229, TS 26.114; RFC 3261

- Security / crypto — TS 33.501, TS 35.205/206/207; RFC 4493 (AES-CMAC)

- NWDAF — TS 23.288, TS 29.520, TS 29.522

- eSIM — GSMA SGP.22, ITU-T E.118, TS 31.102

- SCTP — RFC 4960 + RFC 6458

# 7. Status & licensing

This is reference / research-grade software. The Go core is exercised against the in-tree tester for ~545 test cases. It is suitable for lab interop and conformance trials, feature R&D, spec-aligned tutorials and coursework, and operator experimentation — but it is not currently certified for commercial production use.

MMT Studio is licensed under the GNU Affero General Public License v3.0 or later (AGPL-3.0-or-later). A separate commercial licence is available for use cases not compatible with AGPL-3.0 (for example, embedding the core in a closed-source appliance or operating it as a managed service without publishing modifications). Contact info@makemytechnology.com for terms.

# 8. Glossary

| Term | Meaning |
| --- | --- |
| 5G SA | 5G Standalone — a 5G core that does not depend on a 4G network. |
| NF | Network Function — a logical component of the core (AMF, SMF, UPF, …). |
| gNB | The 5G base station (radio). Emulated by the Tester. |
| UE | User Equipment — a phone or device. Emulated by the Tester. |
| NGAP | Application protocol between gNB and AMF, carried over SCTP. |
| NAS | Non-Access Stratum — signalling between the UE and the core. |
| PDU session | A data connection between a UE and a data network. |
| GTP-U | The tunnelling protocol that carries user data over the network. |
| PFCP | The protocol the SMF uses to control the UPF (N4 interface). |
| DPDK | Data Plane Development Kit — the fast-path library the UPF uses. |
| IMS | IP Multimedia Subsystem — the voice/video (VoNR/ViNR) subsystem. |
| speccheck | The built-in verifier that grounds every spec citation in a local document. |

— End of manual —

MMT Studio 5G  ·  MakeMyTechnology  ·  info@makemytechnology.com